# PGD 1 Semiconductor Materials and Devices

## 50 Examination Questions

### Module 1: Material Physics & Semiconductor Growth

- Define a semiconductor and explain how its electrical conductivity differs from that of a conductor and an insulator.

- Distinguish between elemental semiconductors and compound semiconductors, giving two examples of each.

- Explain why silicon is widely used in semiconductor device fabrication.

- Compare the properties and applications of silicon (Si), germanium (Ge), and gallium arsenide (GaAs).

- What is a crystal lattice? Explain the significance of crystal structure in semiconductor materials.

- Define Miller indices and explain how they are used to identify crystal planes.

- Determine the Miller indices of a crystal plane having intercepts of 2a, 3b, and 1c.

- Explain the difference between covalent, ionic, and metallic bonding and identify the dominant bonding mechanism in silicon.

- Describe the major stages involved in converting metallurgical-grade silicon (MG-Si) into electronic-grade silicon (EG-Si).

- Explain the principle and operation of the Czochralski (CZ) crystal growth method.

- State four advantages and two limitations of the Czochralski method.

- Explain the principles of vapour-phase epitaxy (VPE), liquid-phase epitaxy (LPE), and molecular-beam epitaxy (MBE).

- Compare VPE, LPE, and MBE in terms of growth mechanism, temperature, precision, and applications.

### Module 1: Impurity Control & Doping

- What is semiconductor doping? Explain why doping is necessary in semiconductor device fabrication.

- Differentiate between n-type and p-type semiconductors.

- Explain the difference between donor impurities and acceptor impurities, giving suitable examples.

- Describe the mechanism of dopant diffusion in a semiconductor.

- State and explain Fick's first and second laws of diffusion.

- A semiconductor is doped by diffusion with a constant surface concentration of 1×1020 cm-3. Explain how the impurity concentration varies with depth.

- Explain the principle of ion implantation and identify its major advantages over thermal diffusion.

- Compare diffusion and ion implantation as semiconductor doping techniques.

### Module 2: Energy Bands & Carrier Statistics

- Explain the basic assumptions of the quantum mechanical model of electrons in a semiconductor.

- State the time-independent Schrödinger equation and explain the physical meaning of its terms.

- Explain the basic principle of the Kronig-Penney model and how it leads to the formation of energy bands.

- Distinguish between valence band, conduction band, and forbidden energy gap.

- Explain the difference between a direct band-gap and an indirect band-gap semiconductor.

- What is meant by an intrinsic semiconductor? Derive the expression for intrinsic carrier concentration.

- Explain the difference between intrinsic and extrinsic carrier concentrations.

- Silicon has an intrinsic carrier concentration of approximately 1.0×1010 cm-3at a specified temperature. If the electron and hole concentrations are equal, determine the electron and hole concentrations.

- Explain the Fermi-Dirac distribution function and its significance in semiconductor physics.

- Write the Fermi-Dirac distribution equation and explain what happens to the distribution at absolute zero.

- Define the density of states in a semiconductor and explain its importance in determining carrier concentration.

- Explain how the Fermi energy level changes with doping in n-type and p-type semiconductors.

- An n-type semiconductor has a donor concentration of 5×1016 cm-3. Assuming complete ionization, determine the approximate majority-carrier concentration.

### Module 3: Carrier Transport & Injection Phenomena

- Define carrier mobility and explain the factors that affect electron and hole mobility in semiconductors.

- Explain the mechanisms responsible for carrier scattering in semiconductor materials.

- Derive the expression for drift current density due to electrons and holes.

- A semiconductor has an electron concentration of 2×1016 cm-3, electron mobility of 1350 cm2/V\cdotps, and an electric field of 10 V/cm. Calculate the electron drift current density.

- Explain diffusion current in a semiconductor and state Fick's law.

- State and explain the Einstein relation between carrier mobility and diffusion coefficient.

- Explain velocity saturation and hot-carrier effects in semiconductors.

- What is meant by generation and recombination of charge carriers?

- Explain the Shockley-Read-Hall (SRH) recombination mechanism.

- Define minority-carrier lifetime and explain its significance in semiconductor devices.

- Explain the continuity equation for minority carriers and discuss its application to semiconductor devices.

- What is surface recombination velocity? Explain how semiconductor surface states affect carrier lifetime.

### Module 4: Junctions, Interfaces & Characterisation

- Explain the formation of the depletion region and built-in potential in a p-n junction.

- Derive the ideal Shockley diode equation and explain the effect of forward and reverse bias.

- Differentiate between a Schottky barrier diode and an Ohmic contact. Explain their energy-band characteristics.

- Explain the principles of I-V and C-V characterisation of semiconductor devices and describe how C-V measurements can be used to estimate doping concentration.