7. Electron Concentration

The electron concentration in the conduction band is obtained from:

n=EC∞gC(E)f(E)dE

For a non-degenerate semiconductor, the Fermi-Dirac distribution can be approximated by the Maxwell-Boltzmann distribution:

f(E)≈exp⁡[−E−EFkT]

This gives:

n=NCexp⁡[−EC−EFkT]

or equivalently:

n=NCexp⁡[EF−ECkT]

8. Hole Concentration

The probability that a state is empty is:

1-f(Eq

The hole concentration is:

p=−∞Evgv(E)[1−f(E)]dE

For a non-degenerate semiconductor:

p=NVexp⁡[−EF−EVkT]

p=NVexp⁡[EV−EFkT]

new

Therefore:

n=NCe(EF−EC)/kT

and

p=Nve(Ev−EF)/kT

These equations are fundamental to semiconductor carrier statistics.

9. Position of the Fermi Level

From the electron concentration equation:

n=Nce(EF−EC)/kT

Taking the natural logarithm:

ln⁡(nNC)=EF−ECkT

Therefore:

EF=EC+kTln⁡(nNC)

Similarly, from the hole concentration:

EF=EV−kTln⁡(pNV)

These equations allow the Fermi level to be calculated when carrier concentration is known.

10. Intrinsic Semiconductor

A semiconductor with no intentional donor or acceptor doping is called an intrinsic semiconductor.

new

For an intrinsic semiconductor:

n=p=ni

where ni is the intrinsic carrier concentration.

The intrinsic carrier concentration is:

ni=NcNVexp⁡(−Eg2kT)

where:

Eg=EC−EV

is the band-gap energy.

This equation shows that intrinsic carrier concentration strongly depends on temperature and band-gap energy.

11.Intrinsic Fermi Level

For an intrinsic semiconductor:

n=p=ni

The intrinsic Fermi level Ei is approximately:

Ei=Ec+Ev2+34kTln⁡(mp∗mn∗)

If:

mn∗=mp∗

then:

Ei≈EC+EV2

Thus, the intrinsic Fermi level is approximately located at the middle of the forbidden energy gap.

O211,.

new

12. Mass Action Law

For a semiconductor in thermal equilibrium:

np=ni2

This is called the mass action law.

For an intrinsic semiconductor:

n=p=ni

therefore:

np=ni2

For an n-type semiconductor where≫p:

n≈ND

and:

p=ni2n

Therefore:

| p≈ni2ND |  |
| --- | --- |

For a p-type semiconductor:

p≈NA

and:

n≈ni2NA

13. Effect of Doping on the Fermi Level

new

Doping changes the concentration of charge carriers and therefore shifts the Fermi level.

N-type semiconductor

Donor atoms introduce additional electrons.

The Fermi level moves towards the conduction band.

For an n-type semiconductor:

EF=Ei+kTln⁡(nni)

If donors are fully ionized and:

n≈ND

then:

P-type semiconductor

Acceptor atoms create holes.

The Fermi level moves towards the valence band.

EF=Ei−kTln⁡(pni)

If acceptors are fully ionized:

p≈NA

SO:

14. Charge Neutrality in a Doped Semiconductor For a semiconductor containing electrons, holes, donors and acceptors:

new

p+ND+=n+NA−

where:

。 p=holeconcentration.

· n=electronconcentration

。ND+=ionizeddonorconcentration.

· NA=ionizedacceptorconcentration.

If donors and acceptors are completely ionized:

p+ND=n+NA

This equation is called the charge-neutrality condition.

15. N-Type Semiconductor

When a semiconductor is doped with a donor impurity, it becomes n-type.

Examples of donor impurities in silicon include:

· Phosphorus (P)

· Arsenic (As)

· Antimony (Sb)

A donor contributes an extra electron.

If:

ND≫ni

and all donors are ionized:

n≈ND

Using the mass action law:

np=ni2

we obtain:

new

16. P-Type Semiconductor

When silicon is doped with an acceptor impurity, it becomes p-type.

Examples include:

· Boron (B)

· Aluminium (Al)

· Gallium (Ga)

An acceptor creates a hole.

If:

NA≫ni

then:

p≈NA

and:

n≈ni2NA

17. Fermi Level as a Function of Temperature

Temperature has a major effect on carrier concentration and Fermi level.

There are generally three regions:

Region 1: Freeze-out region

At very low temperature, donor and acceptor atoms are not fully ionized.

Consequently, there are relatively few free carriers.

Region 2: Extrinsic region

At moderate temperature, dopants become fully ionized.

new

For n-type material:

n≈ND

For p-type material:

p≈NA

This is the region normally used in many basic semiconductor calculations.

Region 3: Intrinsic region

At sufficiently high temperature, thermally generated electron-hole pairs become dominant.

Eventually:

n≈p≈ni

The semiconductor behaves increasingly like an intrinsic semiconductor.

18. Worked Example: N-Type Silicon

Consider silicon at 300K with:

ND=1×1016cm−3

Assume:

ni=1×1010cm−3

and complete donor ionization.

Step 1: Electron concentration

Since:

ND≫ni

we have:

n≈ND

Therefore:

UCV.

new

n≈1×1016cm−3

Step 2: Hole concentration

Using:

np=ni2

p=ni2n

Therefore:

p=(1010)21016

p=104cm−3

Thus, electrons are the majority carriers, while holes are the minority carriers.

19. Worked Example: Fermi Level of N-Type Silicon

At 300K, suppose:

ND=1016cm−3

and:

ni=1010cm−3

The Fermi level relative to the intrinsic level is:

EF−Ei=kTln⁡(NDni)

At 300K:

kT≈0.02585eV

Therefore:

EF−Ei=0.02585ln⁡(106)

Since:

new

ln⁡(106)=13.816

we obtain:

EF−Ei≈0.357eV

Thus:

EF≈Ei+0.357eV

The Fermi level has moved upward toward the conduction band.

20. Worked Example: P-Type Silicon

Suppose:

NA=1016cm−3

and:

ni=1010cm−3

At 300K:

p≈NA=1016cm−3

Using the mass action law:

n=ni2p

n=10201016

n=104cm−3

The Fermi level is:

EF−Ei=−kTln⁡(NAni)

Therefore:

Carrler Statistics Notes

1.11)

EF−Ei=−(0.02585)(13.816)

EF−Ei≈−0.357eV

Hence, the Fermi level moves downward toward the valence band.

21. Degenerate and Non-Degenerate Semiconductors

The Maxwell-Boltzmann approximation is valid when the Fermi level is sufficiently far from the band edges.

For a non-degenerate semiconductor:

Ec-EF≫kT

for n-type material.

For a degenerate semiconductor, the doping concentration is very high and the Fermi level can approach or enter the conduction or valence band.

In such cases, the full Fermi-Dirac statistics must be used.

Comparison

| Feature | Non-degenerate | Degenerate |
| --- | --- | --- |
| Doping | Relatively low/moderate | Very high |
| Statistics | Maxwell-Boltzmann approximation | Fermi-Dirac |
| Fermi level | Usually inside band gap | Can enter a band |
| Carrier concentration | Simple exponential equations | More complex |

22. Relationship Between Carrier Concentration and Fermi Level

The electron concentration increases exponentially as the Fermi level approaches the conduction band:

n=Nce(EF−EC)/kT

new

Similarly, hole concentration increases as the Fermi level approaches the valence band:

p=Nve(Ev−EF)/kT

Therefore:

Fermi level closer to Ec → more electrons.

Fermni level closer toEV → more holes.

23. Temperature Dependence of Intrinsic Carrier Concentration

Recall:

ni=NcNVe−Eg/(2kT)

Since:

Nc,NV∝T3/2

we can write approximately:

ni∝T3/2e−Eg/(2kT)

The exponential term is especially important.

As temperature increases:

niincreases rapidly

Therefore, semiconductor conductivity generally increases with temperature because more electron-hole pairs are thermally generated.

24. Important Semiconductor Statistics Formula Sheet

Fermi-Dirac function

new

f(E)=11+e(E−EF)/kT

Electron concentration

n=Nce(EF−EG)/kT

Hole concentration

p=NVe(Ev−Er)/kT

Effective density of states

NC=2(2πmn∗kTh2)3/2

NV=2(2πmp∗kTh2)3/2

Intrinsic carrier concentration

ni=NcNve−Eg/(2kT)

Mass action law

np=ni2

Intrinsic Fermi level

| Ei=EC+EV2+34kTln⁡(mp∗mn∗) |  |
| --- | --- |

N-type Fermi level

EF=Ei+kTln⁡(NDni)

P-type Fermi level

EF=Ei−kTln⁡(NAni)

new

Charge neutrality

p+ND+=n+NA−

25. Summary of Week 5

At the end of this module, the student should be able to:

1. Explain the concept of carrier statistics in semiconductors.

2. Define and use the Fermi-Dirac distribution function.

3. Explain the physical meaning of the Fermi energy level.

4. Calculate the probability of electron occupation of an energy state.

5. Explain the density of states in the conduction and valence bands.

6. Calculate electron and hole concentrations.

7. Calculate intrinsic carrier concentration.

8. Apply the mass action law.

9. Determine the position of the Fermi level.

10. Explain the effect of donor and acceptor doping on the Fermi level.

11. Distinguish between intrinsic, n-type and p-type semiconductors.

12. Explain the effect of temperature on carrier concentration.

13. Distinguish between degenerate and non-degenerate semiconductors.

14. Apply charge-neutrality equations to doped semiconductors.

26. Mid-Module Evaluation

Part A —Short Answer Questions

1. Define carrier statistics.

2. What is the Fermi energy level?

3. State the Fermi-Dirac distribution function.

4. What is the probability that an energy state atE=EFis occupied?

5. Define density of states.

6. What is meant by effective density of states?

7. Distinguish between intrinsic and extrinsic semiconductors.

8. Define majority and minority carriers.

new

9. State the mass action law.

10. Explain why doping changes the Fermi level.

Part B - Calculation Questions

Question 1

At 300K, calculate the probability that an energy state 0.10eV above the Fermi level is occupied.

Use:

k=8.617×10−5eV/K

Question 2

A silicon semiconductor has:

ND=5×1015cm−3

and:

ni=1×1010cm−3

Assuming complete ionization, calculate:

(a) Electron concentration.

(b) Hole concentration.

(c) Identify the majority carrier.

Question 3

For a p-type silicon semiconductor:

NA=2×1016cm−3

and:

ni=1×1010cm−3

Calculate:

(a) Hole concentration.

(b) Electron concentration.

new

(c) Identify the minority carrier.

Question 4

At 300K, an n-type semiconductor has:

ND=1017cm−3

and:

ni=1010cm−3

Calculate the position of the Fermi level relative to the intrinsic Fermi level.

Question 5

Explain, using equations, why increasing temperature causes the intrinsic carrier concentration of a semiconductor to increase.