# LECTURE NOTES ON

# CIRCUIT THEORY I (ELE 712)

# DEPARTMENT OF ELECTRICAL AND ELECTRONIC ENGINEERING

# NIGERIAN DEFENCE ACADEMY(NDA)

# ELEMENTARY SIGNALS

In this chapter, the knowledge of basic elementary signal, its representation and analysis will be discussed.

The subtopics are

A. REPRESENTATION OF DISCRETE TIME SIGNALS

- Graphical representation

- Functional representation

- Tabular representation

- Sequence representation

B. BASIC ELEMENTARY SIGNALS (STANDARD SIGNALS) TYPES

- Unit step function.

- Unit ramp function.

- Unit impulse function

- Sinusoidal function

- Unit parabolic function

- Real exponential function

- Complex exponential function

- Rectangular pulse function

- Signum function

- Sinc function

C. BASIC OPERATIONS ON SIGNALS

- Time shifting

- Time reversal

- Time scaling

- Amplitude scaling

- Signal addition

- Signal multiplication

D. CLASSIFICATION OF SIGNALS

- Continuous time and discrete time signals

- Deterministic and random signals

- Periodic and non periodic signals

- Energy and power signals

- Even and odd signals

INTRODUCTION

Signal constitute an important part of our daily life. Anything that carries information can be called a signal. A signal is defined as a single-valued function of one or more independent variable which contains some information. A signal can also be defined as a time varying physical phenomenon which is intended to convey information. A signal can be a function of time, temperature, pressure, distance etc. if a signal depends on only one independent variable it is called one dimensional signal and if it depends on two independent variables, it is called a two-dimensional signal.

REPRESENTATION OF DISCRETE TIME SIGNAL.

A signal is said to be discrete when it is defined at only discrete instant of time. There are four ways of representing the discrete time signals.

Note: - For discrete time signals, the independent variable is time ‘’n” and it is represented by x(n)

- GRAPHICAL REPRESENTATION.

Considering a signal x(n) with values

X (-2) = -3, x (-1) = 2, x (0) = 0, x (1) = 3, x (2) = 1 and x (3) = 2

The graphical representation of these discrete time signals is as shown figure 1

Figure 1 Graphical representation of signals

- TABULAR REPRESENTATION.

In this type of signal representation, the sampling instant “n” and the signal magnitude are represented in a tabular form.

Using the signal values given in the graphical representation, the tabular representation of the values is shown in table 1

Table 1 Tabular representation of signals

| n | -2 | -1 | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- | --- | --- |
| x(n) | -3 | 2 | 0 | 3 | 1 | 2 |

- FUNCTIONAL REPRESENTATION

In this signal representation, the amplitude of the signal is written against the values of n.

Considering a signal x(n) with values

x(-2) = -3, x(-1) = 2 x(0) = 0, x(1) = 3, x(2) = 1 and x(3) = 2. The functional representation is given as

xn=-3  for n= -22  for n=-10  for n=0

xn=3  for n=11  for n=22 f or n=3

Another example of functional representation is

xn=3n for n≥00 for n<0

- SEQUENCE REPRESENTATION.

Given a finite duration signals used in the functional representation. The sequence representation is

X(n) = (-3,2,0,3,1,2)

The arrow mark denotes the n = 0 term. In the case whereby there was no arrow indicated, then the first term corresponds to n = 0

BASIC ELEMENTARY SIGNAL TYPES

Elementary signals play vital role in the study of signals and systems. They serve as basic building block for the construction of more complex signals. The elementary signals are also called standard signals.

- UNIT STEP FUNCTION.

The step function is the type of elementary function which exist only for positive time and is zero (0) for negative time

The unit step function is a step function that has unity magnitude. The usefulness of the unit step function lies in the fact that if we want a signal to have a value of zero for t<0 and the signal to start at t=0, we only need to multiply the signal with unit step function u(t)

The continuous time unit step function u(t) is defined as

ut=1 for ≥00 for t<0

The shifted unit step function u (t – a) is defined as

ut-a=1 for t≥a0 for t<a

That is, it is zero if (t – a) < 0 and equal to 1 if the argument (t – a) ≥0.

The unit step function is graphically represented in figure 2 (a) and (b).

Figure 2	(a) Unit step function			(b) Delayed unit step function

The discrete time unit step sequence u(n)

un=1 for n≥00 for n<0

For the shifted version of the discrete unit step sequence

un-k=1 for n ≥k0 for t<0

The graphical representation is shown in figure 3(a) and (b)

Figure 3 Discrete (a) Unit step function		(b) Delayed unit step function

- UNIT RAMP FUNCTION.

The continuous unit ramp function r(t) is defined as

rt= t for t≥0 0 for t<0

Therefore r(t) = t u(t).

It is a signal whose amplitude varies linearly and can be obtained by integrating the unit step function.

rt=utdt=dt=t for t≥0

ut=ddtr(t)

The shifted version of the unit ramp signal is given by

rt-a=t-a for t≥a0 for t<a

rt-a=t-au(t-a)

The graphical representation of r(t) and r (t – a) is shown in Figure 4

Figure 4  (a) Unit Ramp function		(b) Delayed Unit Ramp function

The discrete time unit ramp r(n) is defined as

rn=n for n≥00 for t<0

rn=nu(n)

The shifted version of discrete time ramp rn-k

rn-k=n-k for n≥k0 for n<k

rn-k=(n-k)u(n-k)

Graphical representation of r(n) and r (n – 2) is shown in figure 5

Figure 5 Discrete (a) Unit Ramp function		(b) Delayed Unit Ramp function

- UNIT IMPULSE FUNCTION

The continuous time unit impulse function(t) “Dirac” plays an important role in signal analysis. It is defined as

∂(t)=1 for t=00 for t≠0

That is -∞∞∂tdt=1

A delayed version of unit impulse function ∂t-a

∂(t-a)=1 for t=a0 for t≠a

The graphical representation of ∂t and ∂t-a are shown in figure 6

Figure 6 (a) Unit Impulse function		(b) Delayed Unit Impulse function

The unit step function is the integral of unit impulse function.

ut=-∞∞∂tdt

∂t=ddtu(t)

The discrete time unit impulse function d(n) is defined as

∂(n)=1 for n=00 for n≠0

The shifted version of the unit impulse function ∂(n-k) is defined as

∂(n-k)=1 for n=k0 for n≠k

The graphical representation of unit impulse function (discrete) is shown in figure 7

Figure 7 Discrete (a) Unit Impulse function		(b) Delayed Unit Impulse function

- UNIT PARABOLIC FUNCTION.

The continuous time unit parabolic function p(t) is defined as

pt=t22 for t≥00 for t<0

or pt=t22u(t)

The shifted version of the unit parabolic function p(t-a) is defined as

pt-a=(t-a)22 for t≥a0 for t<a

or pt-a=(t-a)22u(t-a)

The graphical representation for p (t) and p (t – a) is illustrated in figure 8(a) and the delayed version in (b)

Figure 8 (a) Unit Parabolic function		(b) Delayed Unit Parabolic function

The unit parabolic function can be obtained by integrating the unit ramp function or the double integral of the unit step function.

pt=utdt=rtdt=tdt=t22for t≥0

The relationship between the parabolic ramp and unit step function is

rt=ddtpt   ut=d2dt2p(t)

The discrete time unit parabolic sequence p(n) is defined as

pn=n22 for n≥00 for n<0

or pn=n22u(n)

the shifted version of the discrete time unit parabolic sequence p (n – k) is defined as

pn-k=(n-k)22 for n≥k0 for n<k

or pn-k=(n-k)22u(n-k)

The graphical representation of unit parabolic p(n) and p (n – 3) is shown in figure 9

Figure 9 (a) Unit Parabolic function			(b) Delayed Unit Parabolic function

- SINUSOIDAL SIGNAL

A continuous time sinusoidal signal is given

xt=Asin(ωt+∅)

Where

A = Amplitude

ω = angular frequency

∅ = phase angle in radians

f = frequency

The time period of a continuous time sinusoidal signal is given by

T = 1f=2πω

Figure 10  Sinusoidal Waveform

The discrete time sinusoidal sequence is given by

xn=Asin(φn+∅) where n = integer.

the period of discrete time sinusoidal sequence is

N = 2πωm

where N and m are integer

Note for a discrete time signal to be periodic the angular frequency ω must be a rational multiple of 2π

The graphical representation of sinusoidal signal (discrete) shown in figure 11

Figure 11 Discrete time sinusoidal waveform

- REAL EXPONENTIAL SIGNAL

A continuous time real exponential signal is defined as

xt=Aeαt where A and α are real

Where A =Amplitude

The exponential gotten depends on the values of a and it has these three conditions

- if α =0 x(t) is constant for all time

- if α is positive i.e. α > 0 the signal grows exponentially

- if α is negative i.e. α < 0 the signal decays exponentially

The waveform of the three conditions is represented in figure 12 (a), (b), and (c).

- (b) 				 (c)

Figure 12 Continous time real exponential signal at (a) α =0, (b) α > 0 and (c) α < 0

The discrete time real exponential sequence is defined as

xn=αn for all n

graphical representation of discrete time real exponential sequence

Figure 13 Discrete time real exponential signal at (a) α =0, (b) α > 0 and (c) α < 0

- COMPLEX EXPONENTIAL SIGNAL

The complex exponential signal is defined as

xt=Aest  s= Complex variable, A= Amplitude

s          = σ+jω

xt    =A e(σ+jω)t

= Aeσt ejωt

= Aeσt (cosωt+jsinωt)

The different waveform for the values of σ and ω are shown in figure 14

- (b)  					(c)

(d)  				(e) 					(f)

Figure 14 Complex exponential signals

The discrete time complex exponential sequence is defined as

xt=an ej(ωont+ø)

= an cosωon+ø+jansinωon+∅

For α=1 the real and imaginary part are sinusoidal

α >1 the amplitude of the sinusoidal grows exponentially

α< 1 the amplitude of the sinusoidal decays exponentially

These statements are represented graphically in figure 15

Figure 15 Discrete complex exponential sequence

- RECTANGULAR PULSE SIGNAL

A rectangular pulse signal is a basic function with a constant amplitude over a specific time duration and zero value elsewhere. The unit rectangular pulse signal is defined as

Figure 16 Graphical representation of the rectangular pulse function

It is widely used as a foundational building block in digital communications, radar systems, and signal processing analysis.

ix  SIGNUM FUNCTION

A signum signal (or signum function, denoted as sgn(t)) is a math function that extracts the sign of a real number, giving an output of +1 for positive values, -1 for negative values, and 0 when the input is zero. The Signum function sgn (t) defined as

sgn(t)1 for t>0-1 for t<0

Graphical representation of unit signum function is shown figure 17

Figure 17  Signum function

Signal Processing: Used to find whether a signal is positive or negative.

Control Systems: Helps systems react differently to positive and negative errors.

- SINC FUNCTION

The sine function Sinc(t) is defined as

Sinc(t) =sintt for-∞<t<∞

The sinc function oscillates with period 2π and decays with increasing t. it is an even function of t

Figure 18 Graphical representation of Sinc function

BASIC OPERATIONS ON SIGNALS

The basic operations on signals are as follows

- TIME SHIFTING

The time shifting of a continuous time signal is mathematically represented by

yt=x(t- to)

Time shifting may result in time advance or time delay. If T is positive, the shift is to the right delaying the signal. If T is negative, the shift is to the left causing signal advance.

Example: let x(t) be signal shown in figure 19 at x(t) = 1 at -2 to 2 but 0 otherwise, find x (t – 2)

Figure 19	 (a) Signal				(b) Delayed version

For discrete time signal, time shifting is denoted as

Y(n) = x (n – N) where N is an integer value

- TIME REVERSAL

The time reversal of a signal is the folding of a signal about the vertical axis or at the line t = 0. It is simply taking a mirror image of the signal about the x axis.

Note: time reversal only affects odd signals. It does not affect even signals.

Example: x (t) be a signal shown in figure 20(a), find x(-t)

Figure 20			(a)

Graphically we can find x (-t) by taking mirror image of x(t) about vertical axis shown in figure 20(b).

(b)

- TIME SCALING

Time scaling is the compression or expansion of signal on its time axis. It increases or decreases the duration of the existence of a signal it is mathematically expressed as

Y (t) = x(at)

If α >1 it results in compression by a factor of α

If α <1 it results in expansion by a factor a

Time scaling is illustrated in figure 21 (a), (b), and (c). (a) is the original signa, (b) is the signal compression and (c) is the signal expansion.

Figure 21  Time Scaling

- AMPLITUDE SCALING

Amplitude Scaling is the amplificatory attenuation of a signal. By using amplitude scaling, we can increase or decrease the strength or gain of the signal which depends on the scaling factor. It is mathematically represented by

y(t) = Ax(t)	where A = scaling factor

the amplitude of y(t) at any instant is equal to A times the amplitude of x(t) at that instant. If A >1 it is AMPLIFICATION and if A < 1 it is ATTENUATION. Figure 22 shows the (a) original signal and (b) Amplification

Figure 22		(a)						(b)

- SIGNAL ADDITION

The sum of two continuous time signals x1(t) and x2(t) can be obtained by adding their values at every instant of time. Similarly, the subtraction of one continuous time signal x2(t) from another signal x1(t) can be obtained by subtracting the values of x2(t) from x1(t) at every instant of time.

To be able to illustrate this, let us graphically in figure 23 show the addition of signal x1(t) with signal x2(t) as well as the subtraction of x1(t) from x2(t).

Figure 23 Signal Addition and Signal Subtraction

- SIGNAL MULTIPLICATION

Signal multiplication is also a basic signal operation. The multiplication of two continuous time signal can be performed by multiplying their values at every instant of time.

Let x1(t) and x2(t) are two signals which are multiplied to create a new signal y(t)

Y(t) = x1(t) * x2(t)

Signal multiplication is graphically represented in Figure 24

Figure 24 Signal Multiplication

CLASSIFICATION OF SIGNALS

Signals that are defined at every instant of time is known as a continuous time signals and are denoted by x(t) while signal that are defined only at discrete instant of time are known as discrete time signals. They are denoted by x(n) where n is an integer.

Both continuous time and discrete time signals may further be classified as

- DETERMINISTIC AND RANDOM SIGNALS

A signal is said to be deterministic if it exhibits no uncertainty of its magnitude and phase at any given instant of time. It has a regular pattern and can be completely represented by mathematical equation of any time.

Figure 25  Deterministic signal

A signal is said to be random if it is characterized by uncertainty about its occurrence. The pattern of such signal is irregular. Random signal cannot be represented by a mathematical equation.

Figure 26  Random Signal

- PERIODIC AND APERIODIC SIGNAL

A signal which repeats itself after a fixed time period is called a periodic signal.it is mathematically represented by

x(t) = x (t + T)

for a discrete signal the mathematical representation is given as

x(m) = (n + N) where N = period

Aperiodic or non-periodic signal does not satisfy the condition

x(t) = x (x + T)

therefore, mathematically a non-periodic signal is represented as

x ≠x (t+T)

for discrete non periodic signal is represented by

x(m) ≠ x (n + N) where n is an integer.

Figure 27  Periodic and Aperiodic signal

- ENERGY AND POWER SIGNAL

ENERGY SIGNAL

A signal is said to be an energy signal if and only if its total energy E is finite (i.e. 0<E<∞)

for energy signal average power = 0

A signal is said to be a power signal if its average power p is finite (i.e. 0<P<∞) and total energy E = ∞

Energy Signal

For Continuous Time Signal (CT)

E = limT→∞t=-TTx(t)2

Discrete Time Signal (DT)

E = limN→∞n=-NN x(n)2

POWER SIGNAL

Continuous Time Signal (CT)

P = limT→∞12T t=-TTx(t).2dt

Discrete Time Signal (DT)

P =limN→∞12N+1n=-NNx(n).2

- EVEN AND ODD SIGNAL

Even (Symmetric) signal

A continuous time signal x(t) is said to be even signal if it satisfies the condition.

x(t) = x(-t) for all t

A discrete time signal x(n) is said to be an even (symmetric) signal if it satisfies the condition

x(n) = x(-n) for all n

even signals are symmetrical about the vertical axis or time origin. Cosine wave is an example of an even signal. Some even signals are shown in figure28

Figure 28 Even Signal

Odd (antisymmetric signal)

A continuous time signal x(t) is said to be an odd (anti symmetric) signal if it satisfies the condition.

x(-t) = -x(t) for all t

a discrete time signal x(n) is said to be an odd signal if it satisfies the condition.

x(-n) = -x(n) for all n

odd signals are antisymmetric about the vertical axis. Sine wave is an example of an odd signal for an odd signal x(t) = 0. Some odd signals are shown in figure 29

Figure 29  Odd Signals

Therefore, any signal x(t) can be expressed as the sum of even and odd components

x(t) = xe (t) + xo(t)

where xe (t) = even components

xo(t) = odd components

DERIVATION OF EVEN AND ODD PARTS OF A SIGNAL

even signal x(t) = x(-t)

odd signal x(-t) = -x(t)

xt=xet+xo(t)

x-t=xe-t+xo(-t)

xet-xo(t)

For the even part

xt+x-t=xet+xot+xet-xot

=2xet

xt+x(-t)2=2xe(t)2

xet=12xt+x-t

For the odd part

xt-x-t=xet+xot-[xet-x0t]

=xet+xot-[xet-x0t]

2xo(t)

xt-x(-t)2= 2xo(t)2

xot=12xt+x-t

For the discrete time signal

xen=12xn+x-n

xon=12xn+x-n

Product of even and odd signal

- The product of two even signals is an even signal.

x1-t=x1t

x2-t=x2(t)

then x-t=x1-tx2(t)

= x1tx2t=xt

Therefore x(t) is an even signal.

- The product of two odd signals is an even signal.

x1-t=x1t

x2-t=-x2(t)

then x-t=x1-tx2(-t)

= [-x1t][-x2t]

=x1tx2t=x(t)

Therefore x(t) is an even signal.

- The product of an even signal and an odd signal equals an odd signal.

If x1(t) is even and x2(t) is odd.

x1-t=x1t

x2-t=-x2(t)

then x-t=x1-tx2(-t)

= [-x1t][-x2t]

-xt

Therefore x(t) is an odd signal.

WORKED EXAMPLES

- Sketch each of the signals

- xn=∂n+∂[n-3]

EXPLANATION

For a unit impulse function, it is guided by the condition

∂n=1 for n=00 for n≠0

And for the shifted version

∂n-k=1 for n=k0 for n≠k

Which means that

The x(n) can only be 1 at n = 0 and n =3 else the value will be 0.

- x(n) = u[n] – [n – 5]

- x(t) = ∂t+2

- x(t) = e-tu(t)

- Express the following as sum of weighted displayed impulses

xn=k2-∞∞ak∂[n-k]

at n=1        xn=∂n-1

at n=2          xn=-2∂[n-2]

at n=3       xn=3∂[n-3]

at n=4       xn=-2∂[n-4]

at n=5           xn=∂[n-5]

Therefore xn=∂n-1-2∂n-2+3∂n-3-2∂n-4]+∂[n-5]

- Express the following sequence as a sum of step functions, i.e., in the form

sn=k2-∞∞aku[n-k]

NOW DO THIS!!!

- Give the discrete signal xn in (a), sketch the signal xn-1 and xn+1

- Consider a signal xn, plot xn-3 and xn+2

Signal advance

Signal advance

y(n) = x(n+2)

y(n) = x(n+2)

Signal delay

Signal delay

y(n) = x(n-3)

y(n) = x(n-3)

29

29

- Consider the signal x(t), plot x(2t) and x(t/2)

(i) x(2t)

t= -0.5, 	x(2x-0.5) 	= x (-1)   = 0

t=0, 		x(2x0)        	= x (0)   = 4

t=0.5,		 x (2x0.5)     	= x (1)   = 4

t=1,		 x(2x1)       	 = x (2)   = 4

t=1.5, 		x (2x1.5)    	 = x (3)   = 0

(i) x(t/2)

t=-2, 	x (-2/2)   	 = x (-1)       = 0

t=-1,	 x (-1/2)   	= x (-0.5)     = 3

t=0, 	x (0/2)     	 = x (0)        = 4

t= 1,	 x (1/2)     	 = x (0.5)     = 4

t=3, 	x (3/2)    	 = x (1.5)     = 4

t=4, 	x (4/2)      	= x (2)        = 4

t=6, 	x (6/2)     	= x (3)        = 0

- Consider a signal x[n] = {0,4,3,2,1,2,3,4,0}, Plot y(n)= x (2n) and x (n/2)

(i) y(n) = x(2n);

y (0) = x(2x0)    = x (0) = 1

y (1) = x(2x1)    = x (2) = 3

y (2) = x(2x2)    = x (4) = 0

y (-1) = x(2x-1) = x (-2) = 3

y (-2) = x(2x-2) = x (-4) = 0

(ii) y(n) = x(n/2);

y (-2) = x (-2/2) = x (-1) = 2

y (-4) = x (-4/2) = x (-2) = 3

y (-6) = x (-6/2) = x (-3) = 4

y (0) = x (0/2)   = x (0) = 1

y (2) = x (2/2)   = x (1) = 2

y (4) = x (4/2)   = x (2) = 3

y (6) = x (6/2)   = x (3) = 4

- Consider the signal  x(t), plot (a) x(2t+2), and (b) x(t/2-2)

To solve this question, we need to follow the precedence rule, that is if Time shifting and Time scaling, time reversal and amplitude scaling occurs in same signal.

Rule:

(a)

(b)

- Sketch the even and odd part of the signal

- Determine even and odd part of the signal xn=cos2n+sin4n

Even Part:         xen=12xn+x-n

xn=cos2n+sin4n

x-n=cos-2n+sin-4n=cos2n-sin4n

xn+x-n=cos2n+sin4n+cos2n-sin4n=  2cos2n

∴xen=cos2n

Odd Part:            xon=12[xn-x-n]

xn=cos2n+sin4n

x-n=cos-2n+sin-4n=cos2n-sin4n

xn-x-n=cos2n+sin4n-cos2n-sin4n= 2sin4n

∴xon=sin4n

- Check whether the following are periodic or not. If periodic, determine fundamental time period.

(i) xt=sin20πt

Solution:

Ω=20π	Fundamental time period 		T=2πΩ=2π20π=110s

Hence x(t) is periodic with period

(ii) xt=2cos2t+5+4sin4t+6

Ω1=2

Fundamental time period	T1=2πΩ1=2π2=π	T1T2=21

T=1T1=2T2=π

Ω2=4

Fundamental time period  T2=2πΩ2=2π4=π2

T1T2=ππ2=2(It is rational number)

Hence x(t) is periodic with period T= πs

(iii) xt=2cos3t+1-4sin⁡(4πt)

Solution:

Ω1=3

Fundamental time period	T1=2πΩ1=2π3

Ω2=4π

Fundamental time period	T2=2πΩ2=2π4π=12

T1T2=2π312=4π3It is irrational number

Hence x(t) is Aperiodic

(iv) xn=sin20πn

Solution:

ω=20π

Fundamental time period	N=2πmω=2πm20π=110m

N=1(taking m=10)

Hence x(n) is periodic with period N=1

(v) xn=cos20n

Solution:

ω=20

Fundamental time period	N=2πmω=2πm20=πm10

N could not be an integer for any value of m.

Hence x(n) is aperiodic

(vi) xn=2cos5πn+5+4sin⁡(6πn+6)

Solution:

ω1=5π

Fundamental time period	N1=2πmω1=2πm5π=2m5

N1=2(taking m=5)

ω2=6π

Fundamental time period	N2=2πmω2=2πm6π=m3

N2=1(taking m=3)

N=LCM (N1,N2)=LCM (2,1) =2

x(n) is Periodic with period N=2s

- Check if it is a power or energy signal

i. xt=e-3tu(t)

Energy E∞=limT→∞-TT|xt|2dt=limT→∞0T|e-3t|2dt= limT→∞0Te-6tdt=limT→∞e-6t-60T

=limT→∞e-6T-6-e-0-6=16<∞                       ∵e-∞=0, e-0=1

Power P∞=lim T→∞12T-TTxt2dt=limT→∞12T0Te-3t2dt= limT→∞12T0Te-6tdt

=  lim    T→∞12Te-6t-60T=limT→∞12Te-6T-6-e-0-6=limT→∞12T16=0       ∵e-∞=0, e-0

∵ 1∞=0

Since energy value is finite and average power is zero, the given signal is an energy signal.

ii.xt=4cos3Ωot

Energy E∞=limT→∞-TT|xt|2dt=limT→∞-TT|4cos3Ωot|2dt=limT→∞-TT16cos23Ωotdt

=limT→∞-TT161+cos6Ωot2dt=limT→∞-TT8dt          ∵-TT8cos6Ωotdt=0

=limT→∞8t-TT=limT→∞16T=∞

Power P∞=lim T→∞12T-TTxt2dt=limT→∞12T-TT4cos3Ωot2dt= limT→∞12T-TT16cos23Ωotdt

=limT→∞12T-TT161+cos6Ωot2dt=limT→∞12T-TT8dt          ∵-TT8cos6Ωotdt=0

=limT→∞12T8t-TT=limT→∞12T16T=8 W

Since energy value is infinite and average power is finite, the given signal is power signal.

(iii)    xn=ejπn4+π2

Energy E∞=limN→∞n=-NNx(n)2=limN→∞n=-NNej(πn4+π2)2=limN→∞n=-NN12=limN→∞2N+1=∞

∵ej(ωn+θ)=1  and n=-NN1=2N+1

Average power P∞=limN→∞12N+1n=-NNx(n)2=limN→∞12N+1n=-NNej(πn4+π2)2

lim=N→∞12N+1n=-NN12=limN→∞12N+12N+1=1

Since energy value is infinite and average power is finite, the given signal is power signal